Introduction

The CTVUE does not have a Recipe Manager but can easily handle recipes’ by using Data Arrays.

This Application note can be considered and addendum to CTVUE-1002.  A method of quickly entering recipe formulas in a table format as well as entering data by calling up a single recipe is covered. An application example using a recipe table:  Boxes coming down a conveyor are to be sorted into one of four sort lanes.  Reject boxes are sorted into one of two reject chutes.  Each sort lane has a different distance with respect to main conveyor, hence different index parameters are needed.  Selecting recipe “new widgits” (String Array) will set sort lane number (1st. Integer Array) , sort lane distance (2nd. Integer Array), of sorter servo axis.  In addition, the reject conveyor can be set by reject lane number (3rd. Integer Array), and reject lane distance (4th. Integer Array)

Lane 4

Lane 3

Lane 2

Lane 1

Main Conveyor

Reject 1

Reject 2

Set up Comms

Modus TCP is used in this example.  Select a unique drive IP address for each drive.  Also select the device type from the drop down menu.  This example renames each drive to something more relevant.

Understanding Arrays

There are five different types of arrays available in CTVUE.  Flag, Integer, Multi, Real, and String arrays.  An array can be from 1 element to 1024 elements high.  Think of an array as a 1 dimension table or column of data of the same type that data can be read/write by pointing to array[element].

Array 1, Element 1

Array 2, Element 1

Array 2, Element 2

Array 2, Element 1024

Array 3, Element 1

Array 3, Element 2

Array 3, Element 1024

Array 1, Element 2

Array 1, Element 1024

Define Tags

For this example, 5 arrays will be defined.  First array is of a String Type, with 1024 strings.  It is imperative to set the storage to retentive.  This will store values in non-volatile memory.

- Define String array

- Define Integer arrays for sort lane number, sort lane distance, reject lane number, and reject lane distance.

- Define Integers for Pointer, Scroll Bar, and destination tags for all drives involved.

- Create Screens
The Recipe_Edit_Load screen allows user to load the selected recipe to the drives.  It also has increment and decrement buttons to step through the recipes.  Another option is to enter the recipe number via a keypad.

The Recipe_TableView screen allows user view and enter multiple recipes.  Two methods in indexing the recipe table is the scroll bar on the left side of the screen or by entering specific table number from keypad by selecting the 0 of 1023 touch area of the screen.

- Create program to load recipe to drives.
The following code is executed when the load recipe button is selected.

- Program drives

The drives IP address has to be set to the same IP as entered in the CTVue configuration.

Each drive requires the same two network mapped registers to receive the Lane number and distance.

Write a program:

Do While true

If Var.Var0=1  Then

Wait For DriveInput.1=on

Index.0.Initiate	'Index0,Incremental,Dist=0.0000 revs,Vel=200 revs/m

Wait For Index.AnyCommandComplete

Dwell For Time 1 'seconds

Index.1.Initiate	'Index1,Absolute,Dist=0.0000 revs,Vel=300 revs/m

Wait For Index.AnyCommandComplete

Wait For DriveInput.1=off

Endif

Loop

Other drive programming considerations are outside of the scope of this example.

- Summary

Enclosed in the zip file along with this document are a  CTVue Configuration file and a Power Tool Pro 4.3 example for an Epsilon EP-P that can be used to better understand this application note.