Software & System Products Division

# MC Controllers ANSI protocol Version 1.3

All Control Techniques drives have the ability to communicate using an ANSI protocol, for this reason the same has been developed for the MC controllers.  This allows the MC controller to communicate with a drive system and also to communicate with the CTIU range of machine interfaces.  This first version is an ANSI slave protocol.  The MC204 has an RS485 port, which may be used.  The MC216 requires an external buffer circuit for RS485.  Note that firmware version 1.42 or later should be used, firmware upgrades can be downloaded to the MC controller using the Motion Perfect Software, the latest version is available from the CT support web site.

Details of the ANSI protocol may be found in the Unidrive documentation, or the Unisoft help file.

The ANSI protocol transfers data between menus and parameters, the MC controllers do not have any menus etc so instead, the table area of the controller is used.  eg menu 11 parameter 25 will read/write to or from table location 1125.

As the CT drives use a fixed point system the table is split up into four areas, Integer, 1 Decimal point, 2 Decimal Points, 3 Decimal points and also an area used by the protocol software. These areas are configurable in the setup area of the program though the following rules should be followed:

Areas should not overlap

The ranges should be contiguous

The menu/parameters must be between 0000 and 9999

The table has 16000 areas so the protocol uses only a small area, leaving lots of room for application software.

Refer to page 5 for protocol set-up information.

## WARNING

Be aware that the data that is entered into the tables must not fall outside the following ranges, or the program will limit the data:

Integer			+32767 	to 	–32768

1 Decimal Point  	+3276.7 	to	-3276.8

2 Decimal Points	+327.67 	to	-327.68

3 Decimal Points	+32.767 	to	-32.768

The protocol does not perform any time-out checks etc, so if the data being sent is critical a watchdog signal should be used to check that data is being sent on a regular basis and if data fails appropriate action taken.

Connections

Port 0 - RS232

Motion perfect interface

Port 1 - RS232 / RS485 (RS485 MC204 only)

User Port

Serial Port 1:

1 - RS485 Data Input/Internal 5v on MC216 Only

2 - RS485 Data Input (Inverted)/Internal 0v on MC216 Only

3 - RS232 Transmit

4 - RS232 Ground

5 - RS232 Receive

6 - RS232 +5v Output

7 - RS485 Data Output (Inverted)/Externally buffered output 2 MC216 Only

8 - RS485 Data Output/Externally buffered input 2 MC216 Only

## Future developments

Soon a DriveLink daughter board will be available for communication to the M’ax, MultiAx and SLM products.  Profibus DP Daughter board is expected to available towards the end of 1999.

A 32 bit version of the MotionPerfect software is currently in development and is expected to be available in 1999.

If you require any further information then please do not hesitate to call Software and Systems Products Division:

Gareth Jones, Motion Software Engineer

+44 (0) 1686 612411

gjones_sspd@compuserve.com

Motion Basic Program

'   Program:    ANSI Slave Protocol Driver Software

'   Module:     ANSI_COMM.BAS

'   Platform:   MC204, MC216 to be checked

'-------------------------------------------------------------------

'   Version:    1.3

'   Author:     Gareth Jones

'   Modified:   09/04/98 GJ

'-------------------------------------------------------------------

'   Copyright 1999 Control Techniques

'

'   Control techniques

'   Software & System Products Division (SSPD)

'   Unit 79, Mochdre Ind. Est.

'   Newtown

'   Powys

'   SY16 4LE

'   gjones_sspd@compuserve.com

'   01686 612411

'====================================================================

'Description

'   Control Techniques use an ANSI Protocol to communicate between a master and

'a drive, this program is designed to mimic a drive to allow the MC Range of

'controllers to be interfaced to a system, and also to allow the use of the

'low cost hi spec CTIU MMI.  This intial version is designed only to respond to

'requests from a master connected to port 2 of the MC204 fitted with the RS485

'option, or Port 2 of the MC216 connected through a RS232 to RS485 external

'converter.  Later vesions may have the ability to function as an ANSI master,

'enquire for details.

'

'The protocol is as follows:

'

'Node Address:

'Each drive or controller on the system has a unique address made up of two

'digits, Group address, and Unit address, the addresses each have a range 1-9.

'Addresses 01, 02, 10, 20, 30 ect are invalid.

'Special ASCII charachters are used to frame the data these are as follows:

'

'Char       ASCII

'----------------

'STX        02

'ETX        03

'EOT        04

'ENQ        05

'ACK        06

'Backspace  08

'NAK        21

'

'Read data from the MC Controller

'--------------------------------

'

'the message to the MC Controller is assembled as follows:

'    |  Address  | Parameter |

'|eot|ga|ga|ua|ua|m1|m2|p1|p2|enq|

'

'ga = group address

'ua = unit address

'm  = menu

'p  = parameter

'

'The controller should reply with:

'    | Parameter |   Data  |

'|stx|m1|m2|p1|p2|d1|...|dn|etx|bcc|

'

'd  = data

'bcc= block checksum

'm  = menu

'p  = paramete

'

'Write data to an MC Controller

'------------------------------

'

'the message to the MC Controller is assembled as follows:

'

'    |  Address  |   | parameter |  data   |

'|eot|ga|ga|ua|ua|stx|m1|m2|p1|p2|d1|...|dn|etx|bcc|

'

'ga = group address

'ua = unit address

'm  = menu

'p  = parameter

'd  = data

'bcc= block checksum

'

'The drive will respond with a single control charachter:

'ACK or NAK

'

'If the address from the master is 00 then all the nodes listen.

'If the address from the master is 10, 20, 30, ... then all the nodes

'which start with the group address will listen ie. 1X, 2X, 3X etc.

'in these modes none of the nodes return an ACK or NAK.

'===============================================================================

'

'===============================================================================

'Set Up - Setup the port and data parameters

'===============================================================================

'

init_var:

'Set ANSI protocol parameters

'Set Port

port=1

'Set the group address

ga = 1

'Set the unit address

ua = 1

' The table is split up into four areas integer, 1 decimal place, 2 decimal

' place and 3 decimal places, these areas can be allocated as required

' but must not overlap, and should be sequential.

' INTEGER +/-32000

int_lo = 500

int_hi = 2000

' 1 DECIMAL PLACE +/- 3200.0

dec1_lo = 2500

dec1_hi = 2999

' 2 DECIMAL PLACES +/- 320.00

dec2_lo = 3000

dec2_hi = 3499

' 3 DECIMAL PLACES +/- 32.000

dec3_lo = 3500

dec3_hi = 4999

' Part of table is used to build charachter strings, this will occupy about 20

' table values, this area should not overlap any of the other table values

char_table = 2000

'Set comms port as 19200 baud, 7 data bits, 1 stop bit, even parity

SETCOM(38400,7,1,2,port)

'set TABLE values

GOSUB set_tab

'===============================================================================

'ansi - read and write using ansi protocol MAIN ROUTEIN

'-------------------------------------------------------------------------------

ansi:

'Get the address from the serial port

GOSUB address_data

IF addrs = FALSE THEN GOTO ansi

'Check if the request is a read or write command

GOSUB read_write

'Get the parameter from the serial port

GOSUB getparam

'Check that the menu & parameter data are within the valid ranges

IF read = 1 THEN

GET #port,gv_key

ENDIF

IF param_ok = FALSE THEN

GOSUB nak

GOTO ansi

ENDIF

'Set the Variable type

IF param >= int_lo AND param <= int_hi THEN param_type = 0

IF param >= dec1_lo AND param <= dec1_hi THEN param_type = 1

IF param >= dec2_lo AND param <= dec2_hi THEN param_type = 2

IF param >= dec3_lo AND param <= dec3_hi THEN param_type = 3

'If the request is a read request then send the data

IF read = 1 THEN GOSUB send_param

'If the request is a write request then get the data from the serial port

IF write= 1 THEN

GOSUB getval

IF gv_retstat = FALSE THEN

GOSUB nak

GOTO ansi

ENDIF

GOSUB bcc

GOSUB bcc_comp

IF bcc_ok = TRUE THEN ' Write value to table

TABLE (param,gv_retval)

GOSUB ack

ELSE

GOSUB nak

ENDIF

ENDIF

GOTO ansi

'-------------------------------------------------------------------------------

'===============================================================================

'address_data - Decode Address

'-------------------------------------------------------------------------------

address_data:

addrs = TRUE

GET #port,gv_key

'look for control charachters

IF gv_key <> 4 THEN GOTO address_fault

'-------------------------------------------------------------------------------

GET #port,gv_key

gv_key=TABLE(gv_key)

'look for group address

IF (gv_key <> ga) AND (gv_key <> 0) THEN GOTO address_fault

'check if broadcast message is being sent

IF gv_key = 0 THEN

broadcast = TRUE

ELSE

broadcast = FALSE

ENDIF

GET #port,gv_key

gv_key=TABLE(gv_key)

'look for group address 2nd transmission

IF (gv_key <> ga) AND (gv_key <> 0) THEN

GOSUB nak

GOTO address_fault

ENDIF

'-------------------------------------------------------------------------------

GET #port,gv_key

gv_key=TABLE(gv_key)

'look for unit address

IF (gv_key <> ua) AND (gv_key <> 0) THEN GOTO address_fault

'check if broadcast message is being sent

IF gv_key = 0 THEN

broadcast = TRUE

ELSE

broadcast = FALSE

ENDIF

GET #port,gv_key

gv_key=TABLE(gv_key)

'look for unit address 2nd transmission

IF (gv_key <> ua) AND (gv_key <> 0) THEN

GOSUB nak

GOSUB address_fault

ENDIF

RETURN

'Check if faults

address_fault:

addrs = FALSE

RETURN

'===============================================================================

'read_write - Decode type of command, read or write

'-------------------------------------------------------------------------------

read_write:

'-------------------------------------------------------------------------------

GET #port,gv_key

TABLE ((char_table + gv_digits),gv_key)

'Check if read or write command by looking for control charachter STX

'Load Menu & Parameter Data

IF gv_key <> 2 THEN

read = 1

write = 0

ELSE

write = 1

read = 0

GET #port,gv_key

gv_digits = gv_digits + 1

TABLE ((char_table + gv_digits),gv_key)

ENDIF

RETURN

'===============================================================================

'===============================================================================

'                               Recieve data

'===============================================================================

'===============================================================================

'===============================================================================

'get_param - get the parameter data from the serial port

'-------------------------------------------------------------------------------

' getparam

'-------------------------------------------------------------------------------

getparam:

' NOTE: A GET statement has already got the first digit from the serial port

gv_sign=1

gv_int=0

gv_dec=0

gv_div=10

gv_count=0

gv_digits=0

IF gv_key>57 OR gv_key<48 THEN

GOTO gvdone

ENDIF

gv_digits = gv_digits + 1

TABLE ((char_table + gv_digits),gv_key)

gv_key=TABLE(gv_key)

i1:

gv_count=gv_count+1

gv_int=gv_int*10+gv_key

integer:

IF gv_count = 4 THEN

GOTO gvdone

ENDIF

GET #port,gv_key

gv_digits = gv_digits + 1

TABLE ((char_table + gv_digits),gv_key)

IF gv_key>57 OR gv_key<48 THEN

GOTO gvdone

ENDIF

gv_key=TABLE(gv_key)

GOTO i1

gvdone:

' Check that four digits have been recieved for the parameter value

' and it is within range

IF gv_count<>4 OR gv_int > dec3_hi OR gv_int < int_lo THEN

param_ok=FALSE

param = 0

ELSE

param_ok=TRUE

ENDIF

param = gv_int

RETURN

'===============================================================================

'getval - Get a decimal number from the serial port

'-------------------------------------------------------------------------------

getval:

'-------------------------------------------------------------------------------

gv_sign=1

gv_int=0

gv_dec=0

gv_div=10

gv_count=0

GET #port,gv_key

gv_digits = gv_digits + 1

TABLE ((char_table + gv_digits),gv_key)

IF gv_key>57 OR gv_key<45 AND gv_key <> 43 THEN

GOTO data_done

ENDIF

gv_key=TABLE(gv_key)

' Decimal point?

checkdp:

IF gv_key=46 THEN

GOTO data_dec

ENDIF

' Minus?

IF gv_key = 45 THEN

gv_sign=-1

GOTO data_int

ENDIF

' Plus?

IF gv_key = 43 THEN

gv_sign= 1

GOTO data_int

ENDIF

GOTO data_i1

data_int:

GET #port,gv_key

gv_digits = gv_digits + 1

TABLE ((char_table + gv_digits),gv_key)

IF gv_key>57 OR gv_key<46 OR gv_key=47 THEN

GOTO data_done

ENDIF

gv_key=TABLE(gv_key)

data_i1:

IF gv_key=46 THEN

GOTO data_dec

ENDIF

gv_count=gv_count+1

gv_int=gv_int*10+gv_key

GOTO data_int

data_dec:

GET #port,gv_key

gv_digits = gv_digits + 1

TABLE ((char_table + gv_digits),gv_key)

IF gv_key>57 OR gv_key<48 THEN

GOTO data_done

ENDIF

gv_key=TABLE(gv_key)

gv_count=gv_count+1

gv_dec=gv_dec+(gv_key/gv_div)

gv_div=gv_div*10

GOTO data_dec

data_done:

IF gv_count=0 THEN

gv_retstat=FALSE

ELSE

gv_retstat=TRUE

ENDIF

gv_retval=(gv_int+gv_dec)*gv_sign

gv_retchar=gv_key

RETURN

'===============================================================================

'bcc - block CHECKSUM calculator FOR recieved bcc

'-------------------------------------------------------------------------------

bcc:

'-------------------------------------------------------------------------------

'Block checksum for recieved information

bcc_check = 0

FOR count = 1 TO gv_digits

bcc_check = bcc_check XOR TABLE((char_table + count))

NEXT count

IF bcc_check < 32 THEN bcc_check = bcc_check + 32

RETURN

bcc_comp:

GET #port,gv_key

IF gv_key = bcc_check THEN

bcc_ok = TRUE

ELSE

bcc_ok = FALSE

ENDIF

RETURN

'===============================================================================

'===============================================================================

'                               transmit data

'===============================================================================

'===============================================================================

'===============================================================================

'nak - Send NAK

'-------------------------------------------------------------------------------

nak:

'-------------------------------------------------------------------------------

'Send NAK (Not Acknowledge)

IF broadcast = TRUE THEN RETURN

PRINT #port, CHR(21);

RETURN

'===============================================================================

'ack - Send ACK

'-------------------------------------------------------------------------------

ack:

'-------------------------------------------------------------------------------

'Send ACK (Acknowledge)

IF broadcast = TRUE THEN RETURN

PRINT #port, CHR(6);

RETURN

'===============================================================================

'Send_param - Send Parameter data

'-------------------------------------------------------------------------------

send_param:

'-------------------------------------------------------------------------------

'Reply to a read command

'Send STX

PRINT #port, CHR(2);

'Send Menu & Parameter

FOR count = 1 TO 4

PRINT #port, (TABLE(char_table+count)-48)[0];

NEXT count

'Send Data

data = TABLE(param)

IF TABLE(param) < 0 THEN

PRINT #port, CHR(45);

data = data * -1

TABLE((char_table + 5), 45)

ELSE

PRINT #port, CHR(43);

TABLE((char_table + 5), 43)

ENDIF

int_val = data

'The data needs to contain a fixed number of integer digits, this calculates

'the zeros put in front of the data so that it is the correct length

'and also checks that the data is within range

IF param_type = 0 THEN

IF int_val > 32767 THEN int_val = 32767

IF int_val < -32768 THEN int_val = -32768

int_data = int_val

ENDIF

IF param_type = 1 THEN

IF int_val > 3276.7 THEN int_val = 3276.7

IF int_val < -3276.8 THEN int_val = -3276.8

int_data = int_val * 10

ENDIF

IF param_type = 2 THEN

IF int_val > 327.67 THEN int_val = 327.67

IF int_val < -327.68 THEN int_val = -327.68

int_data = int_val * 100

ENDIF

IF param_type = 3 THEN

IF int_val > 32.767 THEN int_val = 32.767

IF int_val < -32.768 THEN int_val = -32.768

int_data = int_val * 1000

ENDIF

gv_digits = 5

send_data:

IF int_data < 10 THEN

PRINT #port, "0000";

GOTO send_last

ENDIF

FOR a = 4 TO 1 STEP -1

gv_digits = gv_digits + 1

mult_val = 1

FOR b = 1 TO a

mult_val = mult_val * 10

NEXT b

digit = INT(int_data / mult_val)

'PRINT #6, digit;

'round_var = digit

'GOSUB round

'digit = round_var

int_data = int_data - (mult_val * digit)

round_var = int_data

GOSUB round

int_data = round_var

PRINT #port, digit[0];

TABLE((char_table + gv_digits), (digit + 48))

' Print decimal point

IF a = (param_type) THEN

gv_digits = gv_digits + 1

PRINT #port, CHR(46);

TABLE((char_table + gv_digits), 46)

ENDIF

NEXT a

send_last:

gv_digits = gv_digits + 1

digit = int_data

'PRINT #6, digit

GOSUB round

TABLE((char_table + gv_digits), (digit + 48))

PRINT #port, digit[0];

end_message:

gv_digits = gv_digits + 1

TABLE((char_table + gv_digits), 3)

PRINT #port, CHR(3); ' Send ETX

GOSUB bcc

PRINT #port, CHR(bcc_check); ' Send BCC

RETURN

'===============================================================================

'set_tab - table entries to define the numbers as 0 to 9.

'-------------------------------------------------------------------------------

set_tab:

TABLE(40,40,41,42,43,44)

TABLE(45,45,46,47)

TABLE(48,00,01,02,03,04)

TABLE(53,05,06,07,08,09)

RETURN

round:

digit_int = INT(round_var)

digit_fra = FRAC(round_var)

IF digit_fra > 0.9 THEN digit_int = (digit_int + 1)

round_var = digit_int

RETURN